Bionic Ankle

Nowadays, the need of artificial leg has grown. On one hand, there is the problem of disability. Many people lost their leg, and in need to get a new one. On the other hand, to create human robots, they need to have a bionic ankle. However, most artificial leg does not include mechanical ways to help them walk. There are only few papers that discusses the possibility of a bionic leg that simulates human movement. One example of the bionic leg is Huge Herr’s creation of the bionic leg that increases pacing. However, this bionic leg focuses on helping the patient walk, not about the actual mobility of the real human ankle. This new bionic leg focuses on the simulation of a human ankle, and make the experience of using an artificial leg more humane.

The bionic ankle consists of two motors that can move the leg in all directions, and springs to absorb the impact shock when hitting the ground. The bionic ankle will be able to withstand large impact on the ground and use motors to simulate the muscles in human body to control the ankle. The ankle aims to fully help disabled patients or future human robots to control their ankle easily, just as normal human can do.

The bionic ankle consists of two main systems that simulate the human ankle: the spring system and the motor system. The spring system of the bionic leg acts as hyaline cartilage that absorbs the shock. The motor system acts as muscles in the system all together, allowing a control of moving the foot away and toward the body and a small inclination in left and right.

An average human calf is around 500mm. So, for the upper leg, I estimated it to be a length of 250mm, around one half of the whole leg. The upper leg is where I need to put the two motors. I first need to design two boards that can fix the motors. The motor has a diameter of 89mm. I estimated that I need 10 mm of spacing between the motors for it to not crush into each other. Also, to avoid interference between the two motors, I put one motor on the side of sideboard, and the other motor on the side of another sideboard, with them at two different levels. After calculation, the motor’s center is decided to be 65.5 mm away from the top and bottom side of the board. Moreover, on the other end of the sideboard, there must be a hole for the axle, which is set for 10mm. The lower leg mainly consists of a 3D-printed block, which connects the foot and the upper leg. The lower part of the lower leg is where the tension springs are placed, used to absorb the shock from the leg. To attach the two springs and the tension springs together, there are two connectors for it. In addition to the spring, a gimbal joints is needed to actually connect the foot with the lower leg. The foot is connected to the gimbal joint mentioned in the lower leg part. It is attached a structure, specially designed to withstand immense forces (as it breaks in the first version of the ankle).

For the first model, I used motor MS9015, TTL,-RS485, esp32, gyroscope JY62. The motor MS9015 a rated torque of 1.55 N m, a rated voltage of 24 V, a rated speed of 100 rpm, a max power of 17 W. This motor provides enough power to lift the foot, while also relatively not heavy. The gyroscope JY62 is another important component of the bionic leg because it senses the angle position of the foot. It outputs a stable flow of pitch, roll, and yaw that will be used to PID controller. The steel wire ropes itself cannot provide enough elasticity, so I added springs to the ropes. This, however, made the function between the angular position of the foot and the angle of the motor something that is very hard to simulate. To solve this problem, I implemented a two input to two output PID controller. I designed a model based on the original PID controller. My new model includes identifying the main variable needed to change and focus on changing the main variable, while adding slight deviation to the side variable.