Questions Answers:

What problem does your invention solve?

The bionic ankle solved the problem of leg amputees. It has a 2 DoF, that mimics the movement of a human leg, giving the leg amputees a great experience.

How did you come up with this problem?

As wars and conflicts are going on, more people are in distress. I want to help those people who are in the war. During this process, I learned that there are a lot of people, not only in war, have leg amputation. These triggers me to think about solving this problem.

Who does this problem affect?

It will affect all those leg amputees, and the companies that create bionic legs.

How did you consider the solution to this problem?

To help those people who have leg amputation, I need to help then walk again. I believe having a new leg is the best way to help those people to walk again. A new leg that acts like the old leg is the best way to help those people that I can think of.

Where did you get the materials for your invention?

Some of the materials are 3D printed, bought from web, some model is created and sent to be processed (laser cut).

Who helped you complete the invention, and how did they assist you?

Teachers and parents helped me to complete the inventions. Some of the structures were improved by teachers that are more professional in it. I also need help in tying the strings.

What improvements have you made to this invention?

- I changed some of the parts into parts that have high durability and strength.

- I added a damping to decrease the vibration of motor.

- The strings are tied in a better way such that it takes less time to do it.

- A space is added inside the foot to put the gyroscope inside.

- Some structures near the foot are changed to add more strength and durability to it for it to not break again.

- The strings are added a layer of coating to increase the friction, lowering the sliding of the axle and the string.

Is there room for improvement in your invention?

- The axle can add a guide groove for the string to decrease the sliding of the string even more

- The spring can be changed for better optimization of the numbers.

- The bottom of the foot can add a texture to let the foot work better in different terrains.

Who has used your invention, and what feedback have they provided?

I am only experimenting the invention, and the experiments proved its value.

How do you evaluate your invention?

Out of 10, I give myself a 9. I believe that the direction and main ideas of my invention is on the right track, but I need to refine more details, such as the texture of the foot, the axle’s groove, and programming layer.

Abstract

我第一次看到腿部截肢的问题是在战争中，许多人，甚至是儿童，失去了一条腿。但是当我深入了解截肢者的话题时，我了解到更多的数据：在中国，有2400万人截肢。我真的希望我能用我的发明让这些人重新走路。

我的仿生脚踝旨在帮助这些人重新站起来，因此它在很多方面都模仿了人类的脚踝。这款仿生脚踝具有减震系统，两度自由度（一个上下，一个左右）。

我还做了一些实验来测试脚踝的极限。

我的脚踝可以向四个方向移动。背屈为24.1。跖屈为33.7。内翻为21.9。外翻为14.6。这类似于真正的人类腿。

我还测试了自适应功能，在其中我创建了一个块来表示地面。在给定设定的俯仰和滚动的情况下，脚可以根据这些数字以 25 毫秒的快速反应迅速移动。这是由于PID控制器快速准确。

我通过使用 Solidworks 来描述我的整个结构来开始这个项目。在我制作了虚拟模型后，我检查了它们是否可行，然后我开始构建我的仿生脚踝。这个脚踝由三部分组成（驱动腿、支撑腿和脚）。驱动腿是整个驱动系统所在的位置，有两个电机。支撑腿是减震弹簧所在的位置，它通过万向节连接到脚部。脚是主要的运动部件，它通过拉绳移动。

这是对我的项目的简要描述。

I first saw the problem with leg amputation in war, where many people, even children, lost their leg. But when I got to the bottom of the topic of people with leg amputation, I learned more data: In China, 24 million people have amputation. I really hope that I can use my invention to make these people walk again.

My bionic ankle is designed to help these people get back on their feet, so it mimics a human ankle in many ways. This bionic ankle has a shock-absorbing system, two degrees of freedom (one up and down, one left and right).

I also did some experiments to test the ankle’s limits.

My ankle can move in four directions. The dorsiflexion is 24.1. The plantar flexion is 33.7. The inversion is 21.9. The eversion is 14.6. This is similar to an actual human leg.

I also tested the self-adaptive function, where I created a block to represent the ground. With the set pitch and roll given, the foot can move according to these numbers with a quick reaction speed of 25 ms. This is due to PID controller, which is quick and accurate.

I started this project by using Solidworks to characterize my entire structure. After I made the virtual model, I checked if they were viable, then I started to build my bionic ankles. This ankle is made up of three parts (the driving leg, the supporting leg, and the foot). The drive leg is where the entire drive system is located, there are two motors. The support leg is where the shock spring is located, and it is connected to the foot by a universal joint. The foot is the main moving part of the bionic ankle, which is moved by a rope pull.

This is a brief description of my project.

Statistics

Mass: 2.32 kg

Motor mass: 650 g

Motor Torque: 1.55 Nm, Max Motor Torque: 2.7 Nm

Axle Radius: 0.0118 m

String to Joint: 0.14 m

Torque: 30 Nm Max Torque: 50 Nm

Height: 500mm

Maximum angle for each direction

Dorsiflexion: 10° - 20°

Plantar flexion: 30° - 50°

Inversion: 23°

Eversion: 12°

>10 children a day in 3 months lost their leg

20000 Ukraine experienced 1 amputation

Words

Amputation

Dorsiflexion: 背屈

Plantar Flexion: 跖屈

Inversion: 内翻

Eversion: 外翻

https://www.mayoclinicproceedings.org/article/S0025-6196(12)61642-5/fulltext